Name  Reid Hart

Email  reid.hart.pe@gmail.com

Request for Official Interpretation

Thank you for your official interpretation request concerning 90.1-2022

I.	Section/Table Number

11.5.1

- Background Information

Section 11.5.1, item e, references use of equation 11.5.1, which has a term for net space on the roof (RAnet) available for installation of on-site renewable photovoltaics. There has been some confusion about how to calculate RAnet.  It is defined as: “horizontal projection of roof area available for renewable energy resources not covered by any combination of equipment other than for on-site renewable energy systems, planters, vegetated space, skylights, or occupied roof deck after meeting requirements of Section 10.5.1.”  The purpose of determining RAnet is to find the space available for renewable energy resources, typically photovoltaic collectors, so that a presumed amount of renewable energy resources can be used to meet the energy credit requirement through measure R01. If such space is limited by other uses, then equation 11.5.1 will reduce the energy credit requirement.  Since measure R01 is a renewable energy resource that is above and beyond any resources installed to meet the requirements of Section 10.5.1, then roof area used to meet the 10.5.1 requirement would not be included in the RAnet area.  While a more complete list is included in the interpretation itself, the point is that all areas that are dedicated to other purposes would not be available to provide space available for renewable energy resources beyond the requirements of Section 10.5.1.

- Interpretation

Please provide your interpretation of the requirement(s). Present your interpretation in such a way as to allow for a yes or no answer by the applicable committee. The question below will be presented to the applicable committee for response.

For equation 11.5.1, RAnet is the total building horizontal projection of roof area, less the sum of the areas used by all of the following: • Equipment such as HVAC equipment, piping and ductwork, electrical equipment and wireways, communication equipment and antennas, signage, and other equipment, with the exception of equipment for on-site renewable energy systems. • Planters and vegetated space such as roof gardens, greenhouses, green roof systems, or live roof systems. • Skylights. • Occupied roof deck areas such as pools, pool deck, dining areas, or patios. • Space needed for renewable energy resources to meet the requirements of Section 10.5.1, but not any additional renewable energy resources. • Roof access areas and access walkways to any of the above items and area required for access, cleaning and maintenance of any of the above.  Is this Interpretation correct?

- Supplementary information

Enter any additional information that provides additional context for your question (charts, drawings, etc).

- Question: Is this interpretation correct?

This question will be presented to the applicable project committee for a response

- Answer:

The cognizant Project Committee or Interpretation Committee will provide a yes or no answer.

Yes

- Comments:

The cognizant Project Committee or Interpretation Committee may provide additional comments.

Space for additional renewable energy resources beyond what is required by Section 10.5.1 shall not be subtracted in the calculation of available roof space.